K6A-R20-2M0-20-1L0

- Minimalistic design square emergency luminaire for anti-panic lighting. Recessed mounted.

- Autonomous luminaire (contains battery).

- Dimensions: 80 mm x 80 mm x 50 mm.

- Square light distribution, optimized to avoid dark spots.

- Lumen output in emergency mode: 210 lm.

- Power consumption in standby: 0.2 W.

- Voltage: 220-240V.

- Autonomy: 60 minutes. The luminaire contains lamp data, battery date and type as required according to EN 60598-2.22.

- Battery: 1 x LiFePO4 3,2V 1,5Ah.

- Automatic functional test every week, duration test every 13 weeks, in accordance to EN 50172 and EN 62034.

- Electrical insulation class: class II.

- Photobiological safety IEC/TR 62778: RG1.

- Texture coated aluminum cover, polycarbonate housing, RAL9003 - white / Configurable.

- IP protection level: IP20.

- IK protection level: IK04.

- Ambient temperature: 5°C - 35°C.

- Glowwire: 850°C.

- 5 years warranty on luminaire, light source, driver and battery (for autonomous luminaires).

- Certifications: CE.

- The luminaire has been developed and produced according to standard EN60598-1 in a company that is ISO9001 and ISO14001 certified.