K282/2O-A

IP65 emergency luminaire for final exit illumination. Surface mounted. Autonomous luminaire (contains battery). Dimensions: 359 mm x 180 mm x 100 mm. Mounted on two stainless steel clamps, automatic electrical connection, screwless wire terminals (up to 2.5 mm²), cable gland included, through-wiring possibility, optional accessories for recessed mounting. With battery heater, allowed ambient temperature -20°C to +35°C. Two leds facing towards floor, for 1 lux. Lumen output in emergency mode: 65 lm. Power consumption in standby: 2.0 W. Voltage: 220-230V. The perception distance of the pictogram is  m. Autonomy: 90 minutes. The luminaire contains lamp data, battery date and type as required according to EN 60598-2.22. Battery: 4 x NiMh 1,2V 1,1Ah. Battery lifetime: 10 years. Automatic functional test every week, duration test every 13 weeks, in accordance to EN 50172 and EN 62034. Maintained/non maintained mode selectable, preset for most common application. Electrical insulation class: class II. Photobiological safety IEC/TR 62778: RG0. RAL9018 - papyrus white. IP protection level: IP65. IK protection level: IK10. Ambient temperature: -20°C - 35°C. Glowwire: 850°C. 5 years warranty on luminaire, light source, driver and battery (for autonomous luminaires). Certifications: CE, ENEC. The luminaire has been developed and produced according to standard EN60598-1 in a company that is ISO9001 and ISO14001 certified.